SONAR DISTANCE SIMULATOR

Pulse-Echo Ranging via Matched Filtering and Correlation-Based Delay Estimation

Project Proposal & Technical Documentation

| Field | Detail |
| --- | --- |
| Course | CSE220 — Signals and Systems |
| Team Name | SevenEight |
| Team Members | Aditya Tirtho Roy (2305178) [Team Member 2 — Name & ID] |
| Submitted To | [Course Instructor / Supervisor Name] |
| Institution | Bangladesh University of Engineering and Technology (BUET) |
| Date | August 2026 |

# 1. Introduction

Pulse-echo ranging is the working principle behind sonar, radar, ultrasonic rangefinders, and time-of-flight LiDAR: a short pulse of energy is transmitted, it reflects off a target, and the round-trip travel time of the echo is used to estimate distance. Although the physical medium differs (acoustic, radio, or optical), the underlying signal processing problem is identical — recovering a precise time delay from a noisy, attenuated copy of a known transmitted signal.

This project simulates that entire pipeline in software: generating a transmit pulse, modelling a noisy echo channel, and recovering the delay (and hence distance) using matched filtering and cross-correlation. The project is derived from the course-provided brief: "Simulate sending a short pulse and receiving a delayed echo. Estimate distance from the delay and study how noise and filtering affect the estimate."

Beyond fulfilling the course requirement, this project is directly relevant to our ongoing work in autonomous vehicles (AUV/UAV), where ranging and obstacle-detection sensors are essential components of GPS-denied navigation systems.

# 2. Objectives

- Simulate a realistic pulse-echo sensing pipeline entirely in software using known ground-truth parameters.

- Implement matched filtering / cross-correlation as the core technique for delay estimation.

- Quantify how additive noise (SNR) degrades distance estimation accuracy.

- Study how pulse shape and bandwidth affect range resolution (time-bandwidth trade-off).

- Evaluate the benefit of band-pass filtering as a pre-processing step for robustness.

- Produce a documented, reproducible codebase with clear plots and quantitative evaluation (RMSE vs. SNR, resolution vs. bandwidth).

# 3. Project Scope — Software vs. Hardware

This project is entirely software-based. The course brief explicitly specifies a simulation ("simulate sending a short pulse and receiving a delayed echo"), so no physical sonar transducer, ultrasonic sensor, or microcontroller (e.g. Arduino, STM32) is required for the core deliverable. Everything — the transmitted pulse, the echo channel, the noise, and the receiver — is generated and processed as arrays of numbers in Python.

This keeps the project fully reproducible, gradable from code and plots alone, and removes any dependency on procurement, wiring, or lab access — important given our short timeline.

## Optional Real-World Validation (Stretch Goal, No New Hardware)

If time permits after the core simulation is complete, we may validate the approach acoustically using only a laptop's built-in speaker and microphone: emit a short chirp through the speaker, record the reflection off a wall using the microphone (via Python's sounddevice/pyaudio library), and run the exact same matched-filter pipeline on the recorded audio to estimate real distance. This uses only existing laptop hardware — no purchased components, sensors, or embedded boards — and is entirely optional.

# 4. Theoretical Background

## 4.1 Pulse-Echo Ranging

For a pulse transmitted at t = 0 and its echo received after a round-trip delay τ, the distance to the reflecting target is d = (v · τ) / 2, where v is the propagation speed in the medium (e.g. ~343 m/s for sound in air, or the speed of light for RF/optical ranging). The core estimation problem is recovering τ accurately from a noisy received signal.

## 4.2 Matched Filtering and Cross-Correlation

The optimal way to detect a known pulse shape buried in noise is to correlate the received signal with a copy of the transmitted pulse (the matched filter). The correlation output peaks at the true delay, and the matched filter maximizes the output signal-to-noise ratio for that detection — this is the same principle used in radar, sonar, spread-spectrum communications, and GPS signal acquisition.

## 4.3 Noise Model

Additive White Gaussian Noise (AWGN) will be added to the simulated received signal to represent thermal/electrical noise and environmental interference. Performance will be evaluated across a sweep of signal-to-noise ratios (SNR) to characterize how detection accuracy degrades as SNR decreases.

## 4.4 Range Resolution and Time-Bandwidth Product

A pulse's ability to resolve two closely-spaced targets (range resolution) is governed by its bandwidth — a narrower pulse (wider bandwidth) gives finer resolution but requires a higher sampling rate. We will demonstrate this trade-off directly by comparing narrow rectangular pulses against wider Gaussian-modulated pulses.

# 5. System Architecture / Signal Pipeline

The simulator is structured as five sequential stages, each implemented as an independent, testable Python module:

| Stage | Function |
| --- | --- |
| 1. Transmit Pulse Generator | Produces a known reference pulse (rectangular, Gaussian-modulated sinusoid, or chirp) at a chosen sampling rate. |
| 2. Channel Simulator | Embeds a delayed, attenuated copy of the pulse into a longer silent buffer at a chosen true distance; adds AWGN at a chosen SNR. |
| 3. Receiver / Matched Filter | Cross-correlates the received buffer with the known transmit pulse (matched filter). |
| 4. Delay & Distance Estimator | Locates the correlation peak, converts the corresponding sample delay into an estimated distance. |
| 5. Evaluation & Visualization | Compares estimated vs. true distance across SNR/bandwidth sweeps; generates all report plots. |

# 6. Features to Be Implemented

## Core Features

☐  Configurable transmit pulse generator (rectangular, Gaussian-modulated sinusoid, and linear chirp options)

☐  Channel simulator with adjustable true distance, attenuation, and AWGN noise level (SNR in dB)

☐  Matched filter / cross-correlation based delay and distance estimation

☐  Distance estimation error analysis: RMSE vs. SNR sweep, plotted

☐  Range resolution analysis: estimation accuracy vs. pulse bandwidth

☐  Band-pass filtering pre-processing stage, with before/after robustness comparison

☐  Clear waveform, correlation-output, and error-curve plots for the final report

## Stretch Goals (only if timeline allows)

☐  Multipath / multiple-echo scenario (two or more reflecting surfaces at different distances)

☐  Real acoustic validation experiment using laptop speaker + microphone

☐  Simple interactive parameter interface (matplotlib sliders or CLI) for live demo

# 7. Implementation Plan (Methodology)

## Phase A — Signal Generation & Channel Model

Implement the transmit pulse generator and the channel simulator. Verify correctness first in a noiseless, distortion-free case: given a chosen true delay, confirm the received signal's peak (via simple argmax, no correlation needed yet) matches the expected sample index exactly.

## Phase B — Matched Filter & Noise Study

Implement cross-correlation-based delay detection. Introduce AWGN and sweep SNR from high (e.g. 20 dB) down to low (e.g. -5 dB), recording the estimated vs. true distance at each level. Plot RMSE vs. SNR.

## Phase C — Filtering & Resolution Analysis

Add an optional band-pass filter matched to the pulse's frequency content before correlation; compare detection robustness with and without it at low SNR. Separately, vary pulse width/bandwidth and quantify the smallest resolvable distance between two closely spaced echoes.

## Phase D — Evaluation, Documentation & Demo

Consolidate all results into final plots, write the report, and (if time allows) implement the stretch goals — multipath handling or the real acoustic experiment — as a live demonstration.

# 8. Timeline (4-Week Plan, Target Completion in 3–4 Weeks)

| Week | Focus | Key Tasks | Deliverable |
| --- | --- | --- | --- |
| Week 1 | Setup + Signal & Channel Model | Environment setup; implement pulse generator; implement channel simulator (delay, attenuation); noiseless sanity test | Working pulse + channel simulation, verified against known delay |
| Week 2 | Matched Filter & Noise Study | Implement cross-correlation delay detection; add AWGN; run SNR sweep; plot RMSE vs. SNR | Delay/distance estimator with quantified noise sensitivity |
| Week 3 | Filtering + Resolution Analysis | Add band-pass filtering; compare robustness; vary pulse bandwidth; measure range resolution; start stretch goals if ahead of schedule | Resolution analysis plots; filtering comparison |
| Week 4 | Evaluation, Report & Demo | Finalize all plots; write report; prepare demo/presentation; polish code and repo | Final report, clean codebase, presentation-ready demo |

Note: the team is targeting completion of core features by end of Week 3–4, leaving buffer time before the official 4–5 week deadline for polishing and addressing supervisor feedback.

# 9. Team Roles & Responsibilities

| Member | Primary Responsibilities |
| --- | --- |
| Aditya Tirtho Roy (2305178) | Transmit pulse generation; matched filter / cross-correlation delay estimation; distance conversion; range-resolution & bandwidth analysis |
| [Team Member 2 — Name & ID] | Channel & noise model (AWGN, attenuation); band-pass filtering stage; evaluation metrics (RMSE), plotting, and report compilation |

Both members will jointly handle integration testing, the final evaluation sweep, and the report/presentation — the split above reflects primary ownership, not exclusive responsibility.

# 10. Software & Tools Required

| Tool | Purpose |
| --- | --- |
| Python 3.x | Core implementation language |
| NumPy | Signal generation, array operations, correlation |
| SciPy | Filter design (band-pass), signal processing utilities |
| Matplotlib | Waveform, correlation, and error-curve plots |
| Jupyter Notebook / VS Code | Development environment |
| Git + GitHub | Version control and collaboration |
| sounddevice or pyaudio (optional) | Only needed for the optional real acoustic validation stretch goal |

No paid software, licenses, or specialized lab equipment are required. All tools are free and open-source and run on either team member's personal laptop.

# 11. Deliverables & Evaluation Criteria

- Complete, documented Python codebase (GitHub repository) implementing all core features

- Final written report covering theory, methodology, results, and discussion

- Result plots: waveforms, correlation output, RMSE vs. SNR, resolution vs. bandwidth

- Short presentation / live demo of the simulator

- (If completed) Stretch goal results — multipath handling and/or real acoustic validation

# 12. References

- Oppenheim, A. V., & Willsky, A. S. — Signals and Systems (course reference text)

- Rabiner, L. R., & Gold, B. — Theory and Application of Digital Signal Processing

- North, D. O. (1943) — Original matched filter theory for radar pulse detection

- Course-provided project brief: "Sonar Distance Simulator," CSE220 Project Ideas